SIMATS ENGINEERING

CO3- AT2 - Design Task - Medium

COURSE NAME: Advance Wireless Communication Systems /

5G / 6G Communication Systems

COURSE CODE: ECA56

COURSE OUTCOME COVERED

CO: 3 - Design spectrum access strategies, waveform generation methods, and multiple access techniques for efficient wireless transmission systems. (BL6)

Assessment Tool Weightage: 35%

Short description about assessment: Students design a system / model based on given requirements to assess creativity and application skills.

QUESTIONS: 1 (1 X 100 = 100)		Total Marks: 100 Duration: 3hrs

Develop a wireless communication workflow for AI-assisted healthcare diagnosis. (BL6)

Code of Conduct:

I Vishnuganth G(192512322) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Vishnuganth G

RUBRICS FOR CALCULATION:

| Design Task |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Design | 20% | Demonstrates deep understanding of design objectives, constraints, and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Lacks understanding of the design context |
| Evaluation of Design | 25% | Provides in-depth critique identifying strengths, weaknesses, and design gaps | Identifies key issues with reasonable analysis | Limited critique; mostly descriptive feedback | No meaningful critique provided |
| Justification | 20% | Supports critique with strong reasoning, evidence, and relevant principles | Provides justification with some supporting evidence | Weak or unclear justification | No justification for feedback |
| Suggestions for Improvement | 20% | Offers innovative, practical, and well-justified improvement suggestions | Provides useful suggestions with minor gaps | Suggestions are limited or partially relevant | No constructive suggestions provided |
| Communication | 15% | Communicates feedback clearly, professionally, and engages actively in discussion | Communicates well with minor issues | Communication lacks clarity or engagement | Poor communication and minimal participation |

MATLAB CODE

clc;

clear;

close all;

%% AI-Assisted Healthcare Wireless Communication

% Patient Sensor -> Wireless Channel -> Receiver -> AI Diagnosis

%% 1. Patient physiological data

Fs = 1000;                 % Sampling frequency

t = 0:1/Fs:5;              % 5 seconds

% Simulated ECG-like signal

ecg = 0.8*sin(2*pi*1.2*t) + ...

0.25*sin(2*pi*2.4*t) + ...

0.1*randn(size(t));

%% 2. Patient condition simulation

% 0 = Normal, 1 = Abnormal

patientCondition = 1;

if patientCondition == 0

diagnosisInput = 0.2;

else

diagnosisInput = 0.8;

end

%% 3. Convert physiological data to digital data

threshold = 0.2;

binaryData = ecg > threshold;

%% 4. BPSK Modulation

txSignal = 2*double(binaryData) - 1;

%% 5. Wireless communication channel

SNR_dB = 10;

% Add AWGN noise

rxSignal = awgn(txSignal,SNR_dB,'measured');

%% 6. BPSK Demodulation

receivedBits = rxSignal > 0;

%% 7. Calculate Bit Error Rate

[numErrors, BER] = biterr(binaryData,receivedBits);

fprintf('Wireless Healthcare Communication\n');

fprintf('---------------------------------\n');

fprintf('SNR = %.2f dB\n',SNR_dB);

fprintf('Bit Errors = %d\n',numErrors);

fprintf('BER = %.5f\n',BER);

%% 8. AI-Assisted Diagnosis

% Simple AI-inspired classification

% Feature = average magnitude of physiological signal

feature = mean(abs(ecg));

% Decision threshold

AI_threshold = 0.45;

if feature > AI_threshold

diagnosis = 'ABNORMAL';

else

diagnosis = 'NORMAL';

end

fprintf('\nAI-Assisted Diagnosis: %s\n',diagnosis);

%% 9. Display Communication Workflow

figure('Name','AI Healthcare Wireless Communication');

subplot(3,2,1);

plot(t,ecg);

grid on;

title('1. Patient Sensor Data');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,2,2);

stairs(binaryData(1:200),'LineWidth',1.2);

grid on;

title('2. Digital Data');

xlabel('Sample');

ylabel('Bit');

subplot(3,2,3);

plot(txSignal(1:200));

grid on;

title('3. BPSK Transmitted Signal');

xlabel('Sample');

ylabel('Amplitude');

subplot(3,2,4);

plot(rxSignal(1:200));

grid on;

title('4. Received Signal with Noise');

xlabel('Sample');

ylabel('Amplitude');

subplot(3,2,5);

stairs(receivedBits(1:200),'LineWidth',1.2);

grid on;

title('5. Demodulated Data');

xlabel('Sample');

ylabel('Bit');

subplot(3,2,6);

axis off;

if strcmp(diagnosis,'NORMAL')

text(0.2,0.6,'AI DIAGNOSIS','FontSize',16);

text(0.25,0.4,'NORMAL','FontSize',20);

else

text(0.2,0.6,'AI DIAGNOSIS','FontSize',16);

text(0.25,0.4,'ABNORMAL','FontSize',20);

end

%% 10. Communication workflow diagram

figure('Name','Healthcare Communication Workflow');

axis([0 10 0 6]);

axis off;

text(0.5,5,'Patient Sensors','FontSize',14);

text(3,5,'Wireless','FontSize',14);

text(5.5,5,'Receiver','FontSize',14);

text(8,5,'AI Diagnosis','FontSize',14);

annotation('arrow',[0.18 0.35],[0.75 0.75]);

annotation('arrow',[0.42 0.57],[0.75 0.75]);

annotation('arrow',[0.65 0.80],[0.75 0.75]);

text(0.5,4,'ECG / Vital Signs','FontSize',11);

text(3,4,'BPSK + AWGN','FontSize',11);

text(5.5,4,'Demodulation','FontSize',11);

text(8,4,'Classification','FontSize',11);

text(3.5,2.5,['SNR = ',num2str(SNR_dB),' dB'],...

'FontSize',13);

text(3.2,1.5,['BER = ',num2str(BER)],...

'FontSize',13);

text(6.8,2.0,['Result: ',diagnosis],...

'FontSize',15);

Main parameters

| Parameter | Value |
| --- | --- |
| Sampling frequency | 1000 Hz |
| Duration | 5 s |
| Modulation | BPSK |
| Channel | AWGN |
| SNR | 10 dB |
| AI method | Threshold-based classification |
| Output | Normal / Abnormal |

OUTPUT